# Part I. Optimal communication chain over the ideal channel

## 1. Simulation description

### 1.1 General Model

The objective of the first part of the project is to simulate the optimal communication chain over an ideal channel. The figure below represents the whole communication system in a block-diagram.

Figure 1 - Block diagram of the communication system

It is worth mention that we are applying a baseband equivalent model for the AWGN channel, just simulating the baseband signal without putting it to the high carrier frequency. The discussion about this equivalent could be seen in section 2.

### 1.2 Modulator

For the transmitter side, a random bit-stream is generated as the input of the modulator. Zero padding is carried out to fit the bit-stream to the symbols.

Figure 2 – Constellations of 16QAM, 64QAM, BPSK, QPSK

In this lab, a new modulation mapping function is realised to support different kinds of modulation methods (see section3), however, due to the DVB-S2 set up, BPSK,QPSK,16QAM and 64QAM is applied and compared in this project, which is shown in the following figure.

Modulation is projecting blocks of k bits to M=2^k different waveforms sm(t), and projecting the sm(t) to orthogonal basis, which coefficients {sK } could then be constructed as complex symbols and be shown in constellations. In this lab, as indicated in the previous section, only the complex symbols, say the complex envelop is transferred, without putting it to carrier frequency.

### 1.3 Up-sampling and Down-sampling

Up-sampling with a factor M is carried out to simulate the sampling processing with the specific frequency fs, The factor M is restrained by the symbol frequency. Since a RRC filter is applied (explained in 1.4), the fmax=(1+beta)2Tsym, , in which T refers to the symbol time, Tsym=1/fsym. According to Nquist sampling theory, the sampling rate fs>2*fmax, to avoid alias in digital frequency domain. As M is an integer, M>=2, unless beta=0.

As counter part of the up-sampling step at the transmitter, the down-sampling will allow to get rid of the irrelevant zeros-samples in order to only keep the useful information.

### 1.4 Nyquist Filter (RRC Filter)

Once the sequence of symbols has been generated, the baseband equivalent signal ext can be formed by shaping the symbols thanks to the shaping function g(t).

ext=n=0N-1Ingt-nT

Where N is the number of transmitted symbols and T the period of the shaping function.

The shaping pulse has to be designed according to two criteria

- There should be no Inter-Symbol Interference (ISI) in the generated complex envelope → time domain restriction

- The spectrum of the generated complex envelope should be limited to a well-defined bandwidth in order to avoid Inter-Channel Interference (ICI) → frequency domain restriction

- Mathematically, it can be shown that the ISI is completely avoided when the shaping pulse satisfied

- gnT=1     (n=0)0        (else)

- which is known as the Nyquist ISI criterion. In addition to the rectangular pulse, another popular filter satisfying this criterion is the Raised-Cosine Filter (RCF) whose Fourier Transform is

- Hf=TT21+cosπTβf-1-β2T0  when 0≤f<1-β2T 1-β2T≤f<1+β2T f>1+β2T

- Where β is the roll-off factor, which can range from 0 to 1 and is a measure of the excess bandwidth of the filter. It is this very filter that has been implemented in the project. In particular, in order to draw the best of it, the RC is performed in two steps: namely the shaping pulse g(t) at the transmitter and its matched filter g(-t) at the receiver. As a reminder, the matched filter is the optimal linear filter, situated at the receiver, allowing to maximize the SNR in the presence of AWGN, which is precisely the noise modelled in this project. The Fourier Transform of the shaping pulse is then called the Root-Raised-Cosine Filter (RRCF) and is given by

- Gf=H(f) Since there is also a matched filter at the receiver end.

The filter of different roll-off factor are showed in figure 2 and figure 3, in frequency domain and as impulse response separately.

Figure 3 – Impulse response of RRC Filter

Figure 4 – Frequency Spectrum of RRC Filter

### 1.5 Channel with AWGN

In order to test and compare the different modulation schemes, the best way is to study the potential impact of noise on the BER and SER. To realize it, some Additive White Gaussian Noise (AWGN) is generated and added to the transmitted signal before operations at the receiver take place.

To generate the noise, the amplitude of the noise could be acquired by squaring the per symbol noise power. In this lab, the EbN0 is given, in which Eb refers to the Energy per bit which is demonstrated as below, and N0 is the PSD of the noise.

Etot_s=Ts t=0Tends(t)2,    Eb= Etot_stotal number of bits transfered

Figure 5 – Constellations in receiver of 16QAM, 64QAM, BPSK, QPSK for EbN0=10dB

in which the s(t) refers to the signal transferred in the channel, in this lab it is just the envelope. Then N0= EbEbN0, and PN=2N0*BW, the noise amplitude is 0.5 N0.

Figure 6,7 – BER vs EbN0 for 16QAM,64QAM and BPSK,QPSK

Figure 6 and 7 show the performance of QAM and PSK that are supported by the DVB system. From the figure we could easily conclude that, although higher order modulation rates allowed faster data rates, but it comes with the price of the reduce of resistance for the noise and interference, for example 16QAM has a better BER performance than 64QAM. [TODO]

## 2. Key Points

### 2.1 Simulation Part

1) It is proposed to use the baseband equivalent model of the AWGN channel. Would it be possible to live with a bandpass implementation of the system?

No, this cannot be considered for computing resources reasons: such a simulation would require a too high computing power. In the current baseband equivalent model, the sampling frequency is defined as fs=M*fsymb, where fsymb is typically in the order of the MHz, and M is the upsampling factor. The obtaining of a Narrowband Bandpass Signal would imply a carrier frequency fc much bigger than the actual fsymb, and then a way bigger sampling frequency fs as well. The symbol frequency being already relatively large, it becomes obvious that a bandpass implementation would require too much computing power.

The alternative is then to use the baseband equivalent model. This tip, which cannot be considered in practice, has to be taken into account during the noise generation because the energy of a bandpass signal is equal to the energy of its complex envelope divided by 2. However, a Baseband simulation allows to highlight the same interesting properties and mechanisms as the practical Bandpass communication channel, so this model is totally justified.

2) How do you choose the sample rate in MATLAB?

Sample rate fs is chosen based on the Nquist Sampling theorem. After applying the RRC filter, the signal transferred is band limit to fmax, in this way, fs≥2fmax. As introduce in section 1.3, upsampling of factor M is introduced to simulate the sampling process, normally for non-zero roll-off factor, M should be larger than 2.

3) How do you make sure you simulate the desire EbN0 ratio?

AWGN is generated and added to the channel, by ensuring the noise power, we could make sure simulating the desire EbN0, for details please refers to the 1.5 AWGN part.

4) How do you choose the number of transmitted data packets and their length?

[TODO]

### 2.2 Communication System Part

1) Determine the supported (uncoded) bit rate as a function of the physical bandwidth.

Physical bandwidth is directly restricting the sampling rate fs, since the data transfer through the medium is the samples. Then fsym=fs/M,where M is the up-sampling factor. fbit=fsym*Nbps (bits per second).

2) Explain the trade-off communication capacity/reliability achieved by varying the constellation size.

By increasing the number of bits per symbol (Nbps) we increase the number of symbols as 2Nbps, so we increase the data rate (we need less symbols to forward a same bitstream), but greatly decrease its reliability.

The minimal Euclidean distance and Hamming distance between adjacent symbols always play an important role, while bigger Euclidean distances and associated Hamming distances have a significant effect only for small SNR per bit ratio.

3) Why do we choose the halfroot Nyquist filter to shape the complex symbols?

All the details are given in “Section 1.4. RRC filter”

4) How do we implement the optimal demodulator? Give the optimisation criterion.

Matched filter is used to give the optimal demodulator. Instead of a bank of correlators, we could apply a series of filter hkt=fk-t,where fkt is the basis functions for the modulation. To maximum the SNR, we could deduce (demonstration see Part.3 of the lecture) ht=ks-t, Hf= S*(f), say ht is matched to the signal s(t).

5) How do we implement the optimal detector? Give the optimisation criterion.

The Optimal Detector is implemented by Maximum Likely criterion, smML=maxsmp(r|sm), which is also equivalent to MAP(maximum a posteriori detector) when the transmitted signals are of equal probability.

Then ML criterion reduces to finding the signal sm that is of shortest distance to the received vector r. The detailed description is as following:

- Generating of the complete constellation diagram in order to get a list of the different reference symbols and the associated Gray-Integer

- Comparing each received corrupted symbol with every reference symbol and deduct the one that minimise the Euclidean distance.

## 3. Bonus Research

Out of interest, and disregarding the DVB-S2 set up, our team has also implemented other different modulation methods including K-PSK, K-PAM, K-QAM, 8-Optimized-QAM, and in further compared performance and efficacy of the methods.

### 3.1 Digital Modulation Methods

The current simulation system supports the following modulation schemes:

- Pulse Amplitude Modulation (PAM)

- Phase Shift Keying (PSK)

- Quadrature Amplitude Modulation (QAM)

- Cross - Quadrature Amplitude Modulation (Cross-QAM)

- Optimal - 8 Quadrature Amplitude Modulation (Optimal-8QAM)

It should be noted that, except for the Optimal-8QAM, these modulations can be performed for any number of bits per symbol. To achieve these results, the mapping and demapping functions have been completely reworked by the group members. The figure below shows some examples of constellation diagrams specific to the different modulation schemes supported by the simulation.

Figure 2 – Constellation Symbols of the different supported Modulation Schemes for 8 bits per symbol at the transmitter

### 3.2 Performance Analyse

In particular, considering the Cross-QAM, we choose to implement the '4 symbols per orbit QAM' also called 'Star-QAM' which is one of the most famous flavour of the Cross-QAM